Responsibility Breakdown and Network Tests for AFDX Fuzzer

Goal: Clearly show who is responsible for what at the network level, which test types we will run now, which combined tests we should add later, and the operational/log expectations for each test case. This document is for team presentation and to serve as the basis for creating fuzzer test cases and the oracle.

1) Component schema and responsibilities (network level only)

1.1 Sending ES (ES1)

- Creates and transmits frames including header fields, sequence numbers, and dual transmit on paths A and B. (DONE)

- Produces mutations as required: header mutators, checksum corruptions, length changes, timing / BAG violations, and sequence patterns for multi-frame tasks. (NOT IMMPLEMENTED YET)

- Ensures mutations are sent exactly according to the test pattern and are reproducible/deterministic for regression. (NOT IMMPLEMENTED YET)

1.2 Switch

- Performs filtering and policing at the network level: CRC / FCS checks, frame size validation, destination MAC validity, VL authorization, and BAG policing. (DONE IN V.1 AND NOT SHOULD BE IN V.2)

- Performs forwarding / port mapping and decides drop or forward according to policy. )DONE(

- Must expose counters and logs for drops/forwards per VL and for policing actions. )DONE(

1.3 Receiving ES (ES2)

- Performs network-level validation: frame integrity (FCS), frame size, MAC checks, IP header validation (version and IHL), IP header checksum, disallow fragmentation, IP total length vs UDP length, UDP validation (port mapping, checksum, length), sequence handling (window, gap detection), duplicate detection, and redundancy handling (accept-first, discard-duplicate). (DONE FOR V.2 NEED TO BE TASTED)

- Does not parse payload for these tests; after network validation it hands payload to the upper layer. )NOT IMPLEMENTED)

- Must expose structured logs and metrics containing error/event codes useful for the oracle. (DONE)

2) Test types to run now:

- Single-frame stateless tests (focus on headers and network fields).

- Stateful parser tests (multi-frame sequences to test sequence handling, gaps and duplicates).

- Redundancy path tests (A/B path tests: accept-first / discard-duplicate and late-arrival behavior).

- Timing and BAG tests (send frames too fast or with large jitter).

- Combination tests (multiple simultaneous field errors to reveal problematic interactions).

3) Recommended initial test

ID; Short name; Type; Mutation / Example; Switch expectation; ES2 expectation / Log. T01; FCS corrupt; Mutation; Flip a bit in the FCS; Drop if detected; If it reaches ES2 → Drop; log ERR_CRC. T02; Frame too short; Mutation; Total length < 64 bytes; Drop or mark; If it reaches ES2 → Drop; log ERR_FRAMELEN_SMALL. T03; Frame too long; Mutation; Total length > 1518 bytes; Drop or mark; If it reaches ES2 → Drop; log ERR_FRAMELEN_BIG. T04; Destination MAC mismatch; Mutation; DST MAC not subscribed to VL; Drop per VL mapping; If forwarded → Drop/Ignore; log ERR_MAC. T05; VL authorization invalid; Mutation; VLID not authorized on this port; Drop; If forwarded → Drop; log ERR_VL_AUTH. T06; IP version / IHL invalid; Mutation; IP.version ≠ 4 or IHL < 5; Usually forward; ES2 Drop; log ERR_IP_HDR. T07; IP header checksum corrupt; Mutation; Corrupt IP header checksum; Usually forward; ES2 Drop; log ERR_IP_CKSUM. T08; Fragmentation flag set; Mutation; Set fragment flags; Forward; ES2 Drop per AFDX policy; log ERR_IP_FRAG. T09; IP total length vs UDP mismatch; Mutation; IP.total_len ≠ UDP.length + IP header; Forward; ES2 Drop; log ERR_LENGTH_MISMATCH. T10; UDP destination port invalid; Mutation; dst port not mapped to VL; Forward or drop per config; ES2 Drop; log ERR_UDP_PORT. T11; UDP checksum corrupt; Mutation; Corrupt UDP checksum; Forward; ES2 Drop or mark invalid; log ERR_UDP_CKSUM. T12; Sequence gap; Stateful; Send seq 1 then seq 5 (simulate loss); Forward; ES2 detect gap; log WARN_SEQ_GAP; handle per policy. T13; Duplicate A/B; Stateful; Same frame on A and B with same seq; Forward both; ES2 accept first, discard duplicate; log INFO_DUP_DETECTED. T14; Out-of-window seq; Stateful; Seq outside the allowed window; Forward; ES2 Drop; log ERR_SEQ_OUT_OF_WINDOW. T15; BAG violation; Timing; Send frames faster than BAG interval; Policing: drop or mark; ES2 must not crash; drop excess; log WARN_BAG_VIOL. T16; Large jitter on path B; Redundancy; Large delay on path B; Forward; ES2 accept if arrival within redundancy window; else discard; log WARN_LATENCY. T17; Truncated header; Malformed; Truncate inside IP header; May drop or forward; ES2 safe-fail → Drop; log ERR_TRUNCATED. T18; Payload corrupt beyond headers; Corruption; Corrupt payload only; Forward; ES2 pass to upper layer after header checks; app-level may detect; log INFO_PAYLOAD_CORRUPT if app validation exists. T19; Spoofed source MAC; Security; Replace src MAC; Depends on switch policy; ES2 drop or warn per config; log WARN_SPOOF.

4) Combined tests to add after the basics

- Combine checksum corruption with a sequence gap to see whether parser prioritizes header checksum errors over sequence handling.

- Combine BAG violation with duplicates on A and B to test policing under redundancy load.

- Combine a truncated header on one path with late arrival on the other to test safe-fail behavior during redundancy.

5)  PASS / FAIL / WARNING definitions (simple oracle)

- PASS: Observed actions match expectations on both Switch and ES2, including expected log codes.

- FAIL: Observed actions materially differ from expectations (e.g., Switch drops when forward was expected; ES2 accepts when drop was expected, or expected ERR_* log is missing).

- WARNING: Non-critical unexpected behavior (e.g., correct action but missing detailed log).

6) Minimal required observability / log codes

- ERR_CRC

- ERR_FRAMELEN_SMALL

- ERR_FRAMELEN_BIG

- ERR_MAC

- ERR_VL_AUTH

- ERR_IP_HDR

- ERR_IP_CKSUM

- ERR_IP_FRAG

- ERR_LENGTH_MISMATCH

- ERR_UDP_PORT

- ERR_UDP_CKSUM

- WARN_SEQ_GAP

- ERR_SEQ_OUT_OF_WINDOW

- INFO_DUP_DETECTED

- WARN_BAG_VIOL

- WARN_LATENCY

- ERR_TRUNCATED

- INFO_PAYLOAD_CORRUPT

- WARN_SPOOF

These logs/metrics must be exported in a machine-readable way so the oracle can compare observed vs expected outcomes.

7) Recommended run order and priorities

- Phase 1 — High priority (stability & correctness): T01, T02, T03, T04, T06, T12, T13, T14, T17

- Phase 2 — Medium priority (header checks & timing): T07, T08, T09, T10, T11, T15, T16

- Phase 3 — Low priority (combinations / app-level): T18, T19 and combined tests from section 4